Lab 3 (Task B)

VHDL code for RAM

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

-- Synchronous write / asynchrounous read 32x16 single-port RAM

entity RAM is

Port ( clk : in  STD_LOGIC;

write_en : in  STD_LOGIC;                -- Write enable

Data_In : in  UNSIGNED (15 downto 0);    -- 16-bit data input

Address : in  UNSIGNED (4 downto 0);     -- 5-bit address

Data_Out : out  UNSIGNED (15 downto 0)); -- 16-bit data output

end RAM;

architecture Behavioral of RAM is

type ram_type is array (0 to 31) of UNSIGNED(15 downto 0);

signal ram_inst: ram_type;

begin

-- Asynchronous read

Data_Out <= ram_inst(to_integer(Address));

-- Synchronous write (write enable signal)

process (clk)

begin

if (rising_edge(clk)) then

if (write_en='1') then

ram_inst(to_integer(Address)) <= Data_In;

end if;

end if;

end process;

end Behavioral;

VHDL code for control logic

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity Control is

Port ( clk : in  STD_LOGIC;

rst : in  STD_LOGIC;

nxt : in  STD_LOGIC;

mem_wr : out  STD_LOGIC;

Mem_Addr : out  UNSIGNED (4 downto 0);

r1_en : out  STD_LOGIC;

r2_en : out  STD_LOGIC;

out_en : out  STD_LOGIC;

Mux_Sel : out  STD_LOGIC_VECTOR (1 downto 0));

end Control;

architecture Behavioral of Control is

type fsm_states is (STORE_0, STORE_1, LOAD_R1, LOAD_R2, STORE_N, WAIT_S);

signal state: fsm_states;

signal next_state: fsm_states;

signal done : STD_LOGIC;

signal cnt_en : STD_LOGIC;

signal count : UNSIGNED(4 downto 0);

begin

counter: process (clk) is

begin

if rising_edge(clk) then

if rst = '1' then

count <= "00000";

else

if cnt_en = '1' then

count <= count +1;

end if;

end if;

end if;

end process counter;

state_reg : process (clk) is

begin

if rising_edge(clk) then

if (rst = '1') then

state <= STORE_0;

else

state <= next_state;

end if;

end if;

end process state_reg;

next_states: process(state,rst,nxt,done) is

begin

case state is

when STORE_0 =>

if nxt = '1' then

next_state <= STORE_1;

else

next_state <= state;

end if;

when STORE_1 =>

if nxt = '1' then

next_state <= LOAD_R1;

else

next_state <= state;

end if;

when LOAD_R1 =>

next_state <= LOAD_R2;

when LOAD_R2 =>

next_state <= STORE_N;

when STORE_N =>

next_state <= WAIT_S;

when WAIT_S =>

if nxt ='1' and done = '0' then

next_state <= LOAD_R1;

else

next_state <= state;

end if;

when others =>

next_state <= state;

end case;

end process next_states;

done <= '1' when (count = "11000") else '0';

cnt_en <= nxt when (state = STORE_0) or (state = STORE_1)

else (nxt and (not done)) when state = WAIT_S

else '0';

mem_wr <= '1' when (state = STORE_0)

or (state = STORE_1)

or (state = STORE_N)

else '0';

Mem_Addr <= count when (state = STORE_0)

or (state = STORE_1)

or (state = STORE_N)

else (count - 1) when (state = LOAD_R2)

else (count - 2) when (state = LOAD_R1)

else "00000";

r1_en <= '1' when (state = LOAD_R1)

else '0';

r2_en <= '1' when (state = LOAD_R2)

else '0';

out_en <= '1' when (state = STORE_0)

or (state = STORE_1)

or (state = STORE_N)

else '0';

Mux_Sel <= "10" when (state = STORE_0)

else "11" when (state = STORE_1)

else "00" when (state = STORE_N)

else "00";

end Behavioral;

VHDL Testbench Code

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

ENTITY Fibonacci_tb IS

END Fibonacci_tb;

ARCHITECTURE behavior OF Fibonacci_tb IS

-- Component Declaration for the Unit Under Test (UUT)

COMPONENT Fibonacci

PORT(

clk : IN  std_logic;

rst : IN  std_logic;

nxt : IN  std_logic;

Fib_Out : OUT  std_logic_vector(15 downto 0)

);

END COMPONENT;

--Inputs

signal clk : std_logic := '0';

signal rst : std_logic := '0';

signal nxt : std_logic := '0';

--Outputs

signal Fib_Out : std_logic_vector(15 downto 0);

-- Clock period definitions

constant clk_period : time := 10 ns;

BEGIN

-- Instantiate the Unit Under Test (UUT)

uut: Fibonacci PORT MAP (

clk => clk,

rst => rst,

nxt => nxt,

Fib_Out => Fib_Out

);

-- Clock process definitions

clk_process :processP

begin

clk <= '0';

wait for clk_period/2;

clk <= '1';

wait for clk_period/2;

end process;

-- Stimulus process

stim_proc: process

begin

-- hold reset state for 100 ns.

wait for 100 ns;

nxt <= '0';

rst <= '1';

wait for clk_period*2;

next_loop: LOOP     --loop for next button to toggle with 3 clock periods

nxt <= '1';

wait for clk_period*3;

nxt <= '0';

wait for clk_period*3;

END LOOP next_loop;

wait;

end process;

END;

Simulation

HDL Synthesis

=========================================================================

*                           HDL Synthesis                               *

=========================================================================

Synthesizing Unit <Fibonacci>.

Related source file is "\\userfs\*******\w2k\VHDL\Lab3\Tab3TaskB\Fibonacci.vhd".

Summary:

no macro.

Unit <Fibonacci> synthesized.

Synthesizing Unit <Debouncer>.

Related source file is "\\userfs\*******\w2k\VHDL\Lab3\Tab3TaskB\Debouncer.vhd".

Found 1-bit register for signal <Q1>.

Found 1-bit register for signal <Q2>.

Found 1-bit register for signal <Q0>.

Summary:

inferred   3 D-type flip-flop(s).

Unit <Debouncer> synthesized.

Synthesizing Unit <Datapath>.

Related source file is "\\userfs\*******\w2k\VHDL\Lab3\Tab3TaskB\Datapath.vhd".

Found 16-bit register for signal <R2_Out>.

Found 16-bit register for signal <Fib_Out>.

Found 16-bit register for signal <R1_Out>.

Found 16-bit adder for signal <R1_Out[15]_R2_Out[15]_add_3_OUT> created at line 32.

Summary:

inferred   1 Adder/Subtractor(s).

inferred  48 D-type flip-flop(s).

Unit <Datapath> synthesized.

Synthesizing Unit <RAM>.

Related source file is "\\userfs\*******\w2k\VHDL\Lab3\Tab3TaskB\RAM.vhd".

Found 32x16-bit single-port RAM <Mram_ram_inst> for signal <ram_inst>.

Summary:

inferred   1 RAM(s).

Unit <RAM> synthesized.

Synthesizing Unit <Control>.

Related source file is "\\userfs\*******\w2k\VHDL\Lab3\Tab3TaskB\Control.vhd".

Found 3-bit register for signal <state>.

Found 5-bit register for signal <count>.

Found finite state machine <FSM_0> for signal <state>.

-----------------------------------------------------------------------

| States             | 6                                              |

| Transitions        | 10                                             |

| Inputs             | 2                                              |

| Outputs            | 11                                             |

| Clock              | clk (rising_edge)                              |

| Reset              | rst (positive)                                 |

| Reset type         | synchronous                                    |

| Reset State        | store_0                                        |

| Power Up State     | store_0                                        |

| Encoding           | auto                                           |

| Implementation     | LUT                                            |

-----------------------------------------------------------------------

Found 5-bit adder for signal <count[4]_GND_10_o_add_0_OUT> created at line 1241.

Found 5-bit subtractor for signal <GND_10_o_GND_10_o_sub_23_OUT<4:0>> created at line 1308.

Found 5-bit subtractor for signal <GND_10_o_GND_10_o_sub_25_OUT<4:0>> created at line 1308.

Found 5-bit 3-to-1 multiplexer for signal <_n0053> created at line 100.

Summary:

inferred   2 Adder/Subtractor(s).

inferred   5 D-type flip-flop(s).

inferred   6 Multiplexer(s).

inferred   1 Finite State Machine(s).

Unit <Control> synthesized.

=========================================================================

HDL Synthesis Report

Macro Statistics

# RAMs                                                 : 1

32x16-bit single-port RAM                             : 1

# Adders/Subtractors                                   : 3

16-bit adder                                          : 1

5-bit adder                                           : 1

5-bit subtractor                                      : 1

# Registers                                            : 10

1-bit register                                        : 6

16-bit register                                       : 3

5-bit register                                        : 1

# Multiplexers                                         : 6

1-bit 2-to-1 multiplexer                              : 2

2-bit 2-to-1 multiplexer                              : 1

5-bit 2-to-1 multiplexer                              : 3

# FSMs                                                 : 1